## SECTION 11.  HARDWARE IDENTIFICATION TABLES

Table 7-10. Table of rivets.

| Rivet Number | Description |
| --- | --- |
| AN427 | Rivet, 100§csk. Head steel, monel, copper |
| AN430 | Rivet, round head al. Alloy |
| AN441 | Rivet, tinners Head, steel, ss, monel |
| AN456 | Rivet, brazier head, aluminum alloy |
| AN123151-123750 | Rivet, universal head & 100§ steel, Inconel |
| NAS 1200 | Rivet, solid, 100§ flush shear head |
| NAS 1241 | Rivet, solid, 100§ flush head |
| NAS 1242 | Rivet, solid universal head |
| NAS 1398 | Rivet, blind, protruding head, locked spindle |
| NAS 1399 | Rivet, blind, 100§ csk. Head, locked spindle |

Table 7-10. (continued)

| Rivet Number | Description |
| --- | --- |
| NAS 1738-1739 | Rivet, blind, protruding & flush hd., mech. locked spindle, bulbed |
| NAS1806-1816 | Rivet, hi-shear, flathead., ti. alloy |
| NAS1906-1916 | Rivet, hi-shear, 100° hd., ti. alloy |
| AN124951-125550 | Rivet, solid universal head & 100° csk. head, cres. steel, inconel |
| AN125551-1255700 | Rivet, solid universal head, steel |
| AN426 MS20426 | Rivet, solid, csk. 100° head al. alloy |
| AN470 MS20470 | Rivet, solid, universal head, al. & al. alloy |
| MS9319 | Rivet, solid univ. head, AMS 7233 |
| MS9403 | Rivet, solid, universal head, AMS 5737 |
| MS16535 | Rivet, tubular, oval head |

Table 7-10. (continued)

| Rivet Number | Description |
| --- | --- |
| MS16536 | Rivet, tubular, 100° flat csk. head |
| MS20426 | Rivet, solid, csk., 100° al. alloy |
| MS20427 | Rivet, solid, csk., 100° flush hd., AMS 7233 |
| MS20470 | Rivet, solid, universal head, al. alloy |
| MS20600 | Rivet, blind, pull stem, protruding head |
| MS20601 | Rivet, blind, pull stem, 100°, flush head |
| MS20602-20603 | Rivet, blind, explosive |
| MS20604-20605 | Rivet, blind nonstructural univ. and 100° flush head |
| MS20613-20615 | Rivet, solid, monel, universal hd., steel, ss, brass, copper |
| NAS452-453 | Rivnut, 100° csk. head & flathead |
| NAS454-455 | Rivet blind, al. alloy |

Table 7-10. (continued)

| Rivet Number | Description |
| --- | --- |
| NAS508 | Rivet, universal head, monel |
| NAS1054 | Rivet, hi-shear, protruding head |
| NAS1055 | Rivet, hi-shear, 100° flush head |
| NAS1097 | Rivet, solid, 100° flush shear head, al. Alloy |
| NAS1198 | Rivet, solid, universal head |
| NAS1199 | Rivet, solid, 100° flush head |

Table 7-11. Table of screws.

| Screw MS, AN, or NAS Number | Description |
| --- | --- |
| AN255 | Screw, external relieved body |
| AN500 & 501 | Screw, machine fillister head |
| AN502 & 503 | Screw, machine, fill. Head, drilled, coarse & fine |
| AN504 | Screw, tapping, thread cutting rnd. Head, mach. Thread |

Table 7-11. (continued)

| Screw MS, AN, or NAS Number | Description |
| --- | --- |
| AN505 | Screw, machine, flathead, 82° coarse thread |
| AN506 | Screw, tapping, type F, coarse & fine |
| AN507 | Screw, machine, flathead, 100° |
| AN508 | Screw, machine, round head |
| AN509 | Screw machine, 100° structural |
| AN510 | Screw, machine, flathead, 82° fine thread |
| AN515 & AN520 | Screw, machine, round head |
| AN525 | Screw, washer head |
| AN526 | Screw, machine buttonhead |
| AN530 | Screw-tapping, thread cutting, rnd. hd. |
| AN531 | Screw, tapping, thread forming or cutting, 82° flathead. |

Table 7-11. (continued)

| Screw MS, AN, or NAS Number | Description |
| --- | --- |
| AN535 | Drive screw, round head |
| AN545 | Screw, wood, round head |
| AN550 | Screw, wood, flathead, 82° |
| AN565 | Setscrew, hex. & fluted socket |
| AN115401-115600 | Screws, flat fill. head, steel, .190-.375 |
| AN115601-115800 | Drilled shank Screw flat fill. head, steel, .190-.375 |
| AN115801-116150 | Screw, flat fill. head, steel. .190-.375 |
| AN116901-117080 | Screw, oval fill. head, steel |
| MS9016 | Bushing Screw, plain |
| MS9017 | Bushing Screw, slotted |
| MS9122-9123 | Screw, machine slotted hex. hd. |

Table 7-11. (continued)

| Screw MS, AN, or NAS Number | Description |
| --- | --- |
| MS9177-9178 | Screw, dbl. hex. head, cres. |
| MS9183-9186 | Screw, machine, steel, drilled 12 pt. hd., cad. plate |
| MS9187-9188 | Screw, drilled dbl. hex. head, cres. |
| MS9189-9192 | Screw, machine, steel, 12 pt. hd., black oxide |
| MS9206-9214 | Screw, dbl. hex. ext. washer head, diffused nickel cad. plate |
| MS9215-9223 | Screw, dbl. hex. ext. washer head, diffused nickel cad. plate, drilled |
| MS9281-9291 | Screw, machine, hex. hd., AMS 6322 black oxide |
| MS9292-9302 | Screw, machine, hex. hd., AMS 6322 blk. oxide, drilled |
| MS9316-9317 | Screw, machine, steel slotted hex. head |
| MS9438-9439 | Screw, mach. steel AMS 6304 diffused nickel cad. hex. hd., one hole |
| MS9631-9639 | Screw, mach., hex. hd. one hole, full shank, titanium AMS 4967 |

Table 7-11. (continued)

| Screw MS, AN, or NAS Number | Description |
| --- | --- |
| MS16198 | Screw, wood, slotted RH austenitic corr. res. steel |
| MS16199 | Screw, wood, slot flathead, copper silicone |
| MS16995 | Screw, cap, socket head hex., corr. resisting steel UNC-3A |
| MS16996 | Screw, cap, socket head, hex., corr. resisting steel UNF-3A |
| MS16997 | Screw, cap, socket head, hex., alloy steed cad. UNC-3A |
| MS16998 | Screw, cap, socket head, hex., alloy steel, cad. UNF-3A |
| MS18063-18068 | Setscrew, self-locking, cup, flat, cone pts., steel & stainless |
| MS18211 | Screw, machine, flathead, plastic, nylon |
| MS18212 | Screw, machine, panhead, plastic, nylon |
| MS21090 | Screw, self-lock, panhead, cross recessed |
| MS21207 | Screw, tapping, 100° clk. flathead., steel, cres. steel |

Table 7-11. (continued)

| Screw MS, AN, or NAS Number | Description |
| --- | --- |
| MS21262 | Screw, self-lock, int. wrenching |
| MS21277-21285 | Screw, machine, double hex., ext. washer head |
| MS21286-21294 | Screw, machine, double hex., ext. washer head |
| MS21295 | Screw, self-lock, int. wrenching |
| MS21318 | Screw, drive, round head, Type U |
| MS21342 | Setscrew, fluted socket, cup and flat point |
| MS24615-24618 | Screw, tapping, phillips recessed, pan & 82° flathead Type A |
| MS24619-24622 | Screw, tapping, phillips recessed, pan & 82° flathead Type B |
| MS24623-24626 | Screw, tapping, cross recessed pan & 82° flathead, Type BF or BT |
| MS24627-24630 | Screw, tapping, thread cutting cross recessed pan & 82° flathead, Type F |
| MS24667 & 24671 | Screw, cap socket hd., flat, csk. |

Table 7-11. (continued)

| Screw MS, AN, or NAS Number | Description |
| --- | --- |
| MS24673-24678 | Screw, cap socket hd., flat, drilled |
| MS24693 | Screw, machine, flat csk. head, 100° cross recess |
| MS24694 | Screw, machine, csk. flathead., 100° cross recess, structural |
| MS27039 | Screw, machine, panhead, structural, cross recessed |
| MS35190-35203 | Screw, machine 82° flathead., cross recessed, steel, brass, alum. |
| MS35206-35219 | Screw, machine, panhead, cross recessed, steel, brass, alum. |
| MS35223-35234 | Screw, machine, panhead, slotted, SS steel, steel |
| MS35239-35243 | Screw, machine, flathead., slotted, steel |
| MS35265-35278 | Screw, machine, drilled fillister head, slotted, SS steel, brass, alum. |
| MS35492-35493 | Screw, wood, flathead, cross recessed |
| MS35494-35495 | Screw. wood, flat & round hd., slotted |

Table 7-11. (continued)

| Screw MS, AN, or NAS Number | Description |
| --- | --- |
| MS35914 | Insert, screw, thread, self-tapping |
| MS51017-51047 | Setscrew, hex. socket SS & steel half dog, cone, flat, cup pt. |
| MS51861 | Screw, tapping, type AB, panhead, cross recessed |
| MS51862 | Screw, tapping, type AB, flathead., cross recessed |
| MS51957-51958 | Screw, machine, panhead, cross recessed |
| MS51959-51960 | Screw, machine, flathead, cross recessed |
| MS51963-51966 | Setscrew, hex. socket, cup & flat point |
| MS51973-51974 | Setscrew, hex. socket, cone point |
| MS51975 | Screw, shoulder, socket head |
| MS51976-51977 | Setscrew, hex. socket, half dog point |
| MS51981-51982 | Setscrew, hex. socket, oval point |

Table 7-11. (continued)

| Screw MS, AN, or NAS Number | Description |
| --- | --- |
| NAS220-237 | Screw, brazier hd. phillips recessed |
| NAS384-385 | Screw, oval head, phillips recessed 100°, 82°, steel |
| NAS387-388 | Screw, machine, oval hd., 100°, 82°, steel |
| NAS514 | Screw, machine, 100° flathead., fully threaded, al. steel |
| NAS517 | Screw, 100° flathead, close tol. 160,000 psi |
| NAS548 | Screw, 100° flathead, type B tapping |
| NAS560 | Screw, machine, 100° structural, hi-temp |
| NAS600-606 | Screw, machine, aircraft, panhead phillips recessed, full thr., steel |
| NAS608-609 | Screw, hex. socket cap, plain & self-locking, drilled head |
| NAS623 | Screw, panhead. thr. short, 160,000 psi |
| NAS720 | Screw, panhead, assembled |

Table 7-11. (continued)

| Screw MS, AN, or NAS Number | Description |
| --- | --- |
| NAS1081 | Setscrew, hex. socket, self-locking |
| NAS1096 | Screw, hex. head, recess, full thr. |
| NAS1100 | Screw, machine, panhead, full thread, torq.-set |
| NAS1101 | Screw, machine, flat fill hd. full thread |
| NAS1102 | Screw, machine, 100° flathead. full thr. torq.-set |
| NAS1121-1128 | Screw, machine, flat fill hd., short thread, torq.-set |
| NAS1131-1138 | Screw, machine, pan hd., short thread, torq.-set |
| NAS1141-1148 | Screw, machine, panhead. modified, short thread torq.-set |
| NAS1151-1158 | Screw, machine, 100° flathead., sort thread, torq.-set |
| NAS1161-1168 | Screw, machine, 100° flathead., shear, torq.-set |
| NAS1171-1178 | Screw, panhead., shear, self-lock., torq.-set |

Table 7-11. (continued)

| Screw MS, AN, or NAS Number | Description |
| --- | --- |
| NAS1181-1188 | Screw, flat fill hd., self-locking, torq.-set |
| NAS1189 | Screw, flat 100° hd., full thread, self-locking |
| NAS1190 | Screw, panhead., self-locking, full thread |
| NAS1191 | Screw, flat fill. hd., full thread, self-locking |
| NAS1216 | Screw, panhead hi-torque, full thread |
| NAS1217 | Screw, panhead, hi-torque, short thread |
| NAS1218 | Screw, panhead, hi-torque, short thread |
| NAS1219 | Screw, 100° csk. hd., hi-torque, full thread |
| NAS1220 | Screw, 100° csk. hd., hi-torque, short thread |
| NAS1221 | Screw, 100° clk. hd., hi-torque, long thread |
| NAS1298 | Screw, shoulder, brazier head |

Table 7-11. (continued)

| Screw MS, AN, or NAS Number | Description |
| --- | --- |
| NAS1299 | Screw, shoulder, 100° flathead |
| NAS1300 | Thumbscrew, drilled/undrilled |
| NAS1301 | Screw, panhead, assembled washers phillips recess |
| NAS1351-1352 | Socket Capscrew, hex. head, drilled/undrilled |
| NAS1393 | Screw, 82° flathead, torq-set |
| NAS1402-1406 | Screw, panhead, phillips recess |
| NAS1579 | Screw, panhead., full thread, 1200° F |
| NAS1603-1610 | Screw, flush head, .0312 O.S. |
| NAS1620-1628 | Screw, machine, 100° flat short thread, torq-set |
| NAS1630-1634 | Screw, machine, panhead., short thread, torq.-set |
| NAS1635 | Screw, panhead cross recessed, full thread |

Table 7-11. (continued)

| Screw MS, AN, or NAS Number | Description |
| --- | --- |
| NAS5000-5006 | Screw, panhead, tri-wing recess, short thr., alloy stl. |
| NAS5100-5106 | Screw, panhead, tri-wing recess, short thr., cres. |
| NAS5200-5206 | Screw, panhead, tri-wing recess, short thr., cres. |
| NAS5300-5306 | Screw, fillister head, tri-wing recess, full thr., alloy stl. |
| NAS5400-5406 | Screw, fillister hd., tri-wing recess, full thr., cres. |
| NAS5500-5506 | Screw, fillister hd., tri-wing recess, full thr., titanium |
| NAS5600-5606 | Screw, 100° head, tri-wing recess, full thr., alloy stl. |
| NAS5700-5706 | Screw, 100° head, tri-wing recess, full thr., cres. |
| NAS5800-5806 | Screw, 100° head, tri-wing recess, full thr., titanium |
| NAS5900-5903 | Screw, hex. Head, tri-wing recess, full thr., alloy stl. |
| NAS6000-6003 | Screw, hex. Head, tri-wing recess, full thr., cres. |

Table 7-11. (continued)

| Screw MS, AN, or NAS Number | Description |
| --- | --- |
| NAS6100-6103 | Screw, hex head, tri-wing recess, full thr., titanium |
| NAS6500-6506 | Screw, 100° oval hd., tri-wing recess, full thr., cres. |
| NAS6900-6904 | Screw, panhead, tri-wing recess, full thr., cres. |

Table 7-12. Table of bolts.

| Bolt Number | Description |
| --- | --- |
| AN3-20 | Bolt, machine |
| AN21-36 | Bolt, clevis |
| AN42-49 | Bolt, eye |
| AN73-81 | Bolt, machine, drilled |
| AN173-186 | Bolt, aircraft Close tolerance |
| AN774 | Bolt, flared tube |
| AN775 | Bolt, universal fitting |

Table 7-12. (continued)

| Bolt Number | Description |
| --- | --- |
| AN148551-149350 | Bolt, socket head, 6-hole drilled, .190-.625 |
| AN101001-101900 | Bolt, hex, steel, head |
| AN101901-102800 | Bolt, hex., drilled shank, steel |
| AN102801-103700 | Bolt. Drilled hex. Head, (one hole), steel |
| AN103701-104600 | Bolt, drilled hex. Head, steel, (six holes) |
| AN104601-105500 | Bolt, hex. Head, corrosion-resistant steel |
| AN105501-106400 | Bolt, hex. Head, drilled shank, corrosion-resistant steel |
| AN106401-107300 | Bolt, hex., drilled head, (one holes), corrosion-resistant steel |
| AN107301-108200 | Bolt, hex., drilled head, (six holes), corrosion-resistant steel |
| MS9033-9039 | Bolt, machine 12pt. Head, 130,000 psi min. T.S. |
| MS9060-9066 | Bolt, machine 12pt. Double hex. 130,000 psi min. T.S. ext. washer head, drilled |

Table 7-12. (continued)

| Bolt Number | Description |
| --- | --- |
| MS9088-9094 | Bolt, machine, steel, drilled 12 pt. head |
| MS9110-9113 | Bolt, machine, double hex., ext. washer head, close tolerance |
| MS9146-9152 | Bolt, steel, 12 pt. hd. black oxide 125,000 psi min. T.S. |
| MS9157-9163 | Bolt, steel, 12pt. hd. black oxide 125,000 psi min. T.S. |
| MS9169-9175 | Bolt, steel, 12 pt. drilled hd., black oxide 125,000 psi min. T.S. |
| MS9224 | Bolt, 12 pt. head, heat resistant |
| MS9397-9402 | Bolt, tee head, AMS 6322, chamfered cad. pl. |
| MS9432-9437 | Bolt, tee head AMS 5735 chamfered |
| MS9440-9448 | Bolt, mach. steel. AMS 6304 diffused nickel cad. hex. hd., 3 holes |
| MS9449-9459 | Bolt, mach. steel, AMS 6304 diffused nickel cad., hex. head |
| MS9487-9497 | Bolt, mach. hex. hd. full shank, AMS 5731 |

Table 7-12. (continued)

| Bolt Number | Description |
| --- | --- |
| MS9498-9508 | Bolt, mach. hex. hd., 1 hole, full shank |
| MS9516-9526 | Bolt, mach., steel AMS 6322 cad. 1 hole hex. hd. |
| MS9527-9537 | Bolt, mach., steel AMS 6322 cad. 1 hole hex. hd. |
| MS9554-9562 | Bolt, mach., dbl. hex. ext. wash. hd., PD shank, AMS 5731 |
| MS9563-9571 | Bolt, mach., dbl. hex. ext. wash. hd. drilled, AMS 5731 |
| MS9572-9580 | Bolt, mach., dbl. hex. ext. wash. hd., drilled, PD shank AMS 5731 silver plated |
| MS9583-9591 | Bolt, mach., hex. hd. 6 holes full shank, AMS 5731 |
| MS9622-9630 | Bolt, mach., hex. hd. 1 hole, PD shank, titanium AMS 4967 |
| MS9641-9648 | Bolt, mach., hex. hd., 1 hole, full shank titanium AMS 4967 |
| MS9649-9652 | Bolt, mach., hex. hd. full shank, titanium AMS 4967 |
| MS9676-9679 | Bolt, mach., dbl. hex. ext. wash. hd., cup washer locked, cres. AMS 5731 |

Table 7-12. (continued)

| Bolt Number | Description |
| --- | --- |
| MS9680-9683 | Bolt, mach., dbl. hex. ext. wash. hd., cup washer locked, steel AMS 6322 cad. |
| MS9685-9693 | Bolt, mach., hex. hd. 1 hole, PD shank, steel AMS 6304 diffused nickel cad. |
| MS9694-9702 | Bolt mach. dbl. hex. ext. wash. hd. AMS 5708 |
| MS9703-9711 | Bolt, mach., dbl. hex. ext. wash. hd., drilled, AMS 5708 |
| MS9712-9720 | Bolt, mach. dbl. hex. ext. wash. hd. drilled, AMS 5708 silver plate |
| MS9730-9738 | Bolt, mach., dbl. hex. ext. wash. hd. PD shank, cres. AMS 5643 |
| MS9739-9747 | Bolt, mach. dbl. hex. est. wash, hd. drilled, PD shank, cres. AMS 5643 |
| MS9748-9756 | Bolt, mach. dbl. hex. ext. wash. hd. PD shank, titanium AMS 4967 |
| MS9757-9765 | Bolt, mach., dbl. hex. ext. wash. hd., PD shank, drilled, titanium AMS 4967 |
| MS9781-9791 | Bolt, hex. hd., mach. full shank, AMS 5643 |
| MS9792-9802 | Bolt, mach., hex. hd. 1 hole, full shank, AMS 5643 |

Table 7-12. (continued)

| Bolt Number | Description |
| --- | --- |
| MS9803-9813 | Bolt, mach., hex. Hd. 1 hole, full shank, AMS 5643 |
| MS9814-9824 | Bolt, mach., hex. Hd. 1 hole, PD shank, AMS 5643 |
| MS9883-9891 | Bolt, mach., dbl. Hex. Ext. wash. Hd., AMS 5616 |
| MS9892-9900 | Bolt mach., dbl. Hex. Ext. wash. Hd., AMS 5616 drilled |
| MS9912-9920 | Bolt, mach., dbl. Hex. Ext. wash. Hd., PD shank, steel AMS 6322 cad. |
| MS9921-9929 | Bolt, mach., dbl. Hex. Ext. wash hd. PD shank, steel AMS 6322 cad. Drilled |
| MS9930-9938 | Bolt, mach., dbl. Hex. Ext. wash. Hd., full shank, steel AMS 6322 cad. |
| MS9939-9946 | Bolt, mach., dbl. Hex. Ext. wash. Hd., drilled, full shank, steel AMS 6322 cad. |
| MS20004-20024 | Bolt, int. wrench, 160 KSI |
| MS20033-20046 | Bolt, machine, hex. Head, 1200 °F |
| MS20073-20074 | Bolt, machine, aircraft, drilled hd., fine & coarse thr. |

Table 7-12. (continued)

| Bolt Number | Description |
| --- | --- |
| MS21091-21093 | Bolt, self-lock., 100° flush head, cross recessed |
| MS21094-21095 | Bolt, self-lock., hex. head |
| MS21096-21097 | Bolt, self-lock., panhead, crass recessed |
| MS21098-21099 | Bolt, self-lock., 12 pt. ext. wrenching |
| MS21250 | Bolt, 12 pt., ext. wrenching |
| NAS144-158 | Bolt, internal wrenching, steel, 1/4-28 thru 1-1/8-12 |
| NAS333-340 | Bolt, 100°, close tolerance, hi-strength |
| NAS428 | Bolt, adjusting, crowned hex. hd. |
| NAS464 | Bolt, shear, close tolerance |
| NAS501 | Bolt, hex. head, drilled & undrilled |
| NAS551 | Bolt, universal fitting |

Table 7-12. (continued)

| Bolt Number | Description |
| --- | --- |
| NAS563-572 | Bolt, full thread, fully identified head |
| NAS583-590 | Bolt, 100° head, hi-torque, close tol. 160,000 psi |
| NAS624-644 | Bolt, twelve point external wrench, 180000 psi |
| NAS653-658 | Bolt, hex. head, close tolerance, ti. alloy |
| NAS663-668 | Bolt, full thread, fully identified head |
| NAS673-678 | Bolt, hex. head, close tolerance, ti. alloy |
| NAS1003-1020 | Bolt, machine, hex. head |
| NAS1053 | Eye Bolt Assembly, Shoulder nut |
| NAS1083 | Bolt, 100° flathead, titanium alloy |
| NAS1103-1120 | Bolt, machine, hex. head |
| NAS1202-1210 | Bolt, 100° phil. recessed, close tolerance, 16,000 psi |

Table 7-12. (continued)

| Bolt Number | Description |
| --- | --- |
| NAS1223-1235 | Bolt, self-locking, hex. head 250 °F |
| NAS1236 | Bolt, universal, Tube-end, flareless |
| NAS1243-1250 | Bolt, 100° head, hi-torq. 1600 psi |
| NAS1253-1260 | Bolt, 100° head, flush hd., .0312 O.S. hi-torque |
| NAS1261-1270 | Bolt, hex. head, short thread |
| NAS1271-1280 | Bolt, 12 point hd., external wrenching |
| NAS1297 | Bolt, shoulder, hex. head |
| NAS1303-1320 | Bolt, hex. head, close tolerance, 160,000 psi |
| NAS1414-1422 | Lock bolt, shear 100° head, all. steel |
| NAS1424-1432 | Lock bolt, shear protruding head, steel |
| NAS1503-1510 | Bolt, 100° flush head, hi-torq. |

Table 7-12. (continued)

| Bolt Number | Description |
| --- | --- |
| NAS1516-1522 | Lock Bolt, 100° head, pull type, al. Alloy |
| NAS1578 | Bolt, shear panhead, 1200 °F |
| NAS1580 | Bolt, tension, flush hd., 1200 °F |
| NAS1581 | Bolt, shear reduced 100 °F flush head, 1200 °F |
| NAS1586 | Bolt-tension, 1200 °F, 12 point, external wrenching |
| NAS1588 | Bolt, tension, flush hd., 1200 °F |
| NAS1703-1710 | Bolt, 100° head, .0156 O.S. shank, |
| NAS2005-2012 | Bolt lock, protruding head, ti. Alloy |
| NAS2105-2112 | Bolt, lock, 100° head, ti. Alloy |
| NAS2206-2210 | Bolt, lock, stump type, protruding head, ti. Alloy |
| NAS2306-2310 | Bolt, lock, stump type, 100° head, ti. Alloy |

Table 7-12. (continued)

| Bolt Number | Description |
| --- | --- |
| NAS2406-2412 | Bolt, lock, shear protruding head, ti. alloy |
| NAS2506-2512 | Bolt, lock, 100°head, ti. alloy |
| NAS2606-2612 | Bolt, lock, shear protruding head, ti. alloy |
| NAS2706-2712 | Bolt, lock, shear 100° head, ti. alloy |
| NAS2803-2810 | Bolt, lock, 100° hd., torq-set 180,000 psi |
| NAS2903-2920 | Bolt, hex. head, . 0156 O.S. shank, 160,000 psi |
| NAS3003-3020 | Bolt, hex. head, . 0312 O.S. shank, 160,000 psi |
| NAS3103-3110 | Bolt, U type |
| NAS3203-3210 | Bolt, hook |
| NAS3303-3305 | Bolt, U strap type |
| NAS4104-4116 | Bolt, 100° head, tri-wing recess, long thr., alloy stl. |

Table 7-12. (continued)

| Bolt Number | Description |
| --- | --- |
| NAS4204-4216 | Bolt, 100°head, tri-wing recess, long thr., cres. |
| NAS4304-4316 | Bolt, 100° head, tri-wing recess, long thr., titanium |
| NAS4400-4416 | Bolt, 100° head, tri-wing recess, short thr., alloy stl. |
| NAS4500-4516 | Bolt, 100° head, tri-wing recess, short thr., cres. |
| NAS4600-4616 | Bolt, 100° head, tri-wing recess, short thr., titanium |
| NAS4703-4716 | Bolt, 100° reduced, tri-wing recess, short thr., alloy stl. |
| NAS4803-4816 | Bolt, 100° reduced, tri-wing recess, short thr., cres. |
| NAS4903-4916 | Bolt, 100° reduced, tri-wing recess, short thr., titanium |
| NAS6203-6220 | Bolt, hex. head, short thread, alloy steel |
| NAS6303-6320 | Bolt, hex. head, short thread, cres. |
| NAS6403-6420 | Bolt, hex. head, short thread, titanium |

Table 7-12. (continued)

| Bolt Number | Description |
| --- | --- |
| NAS6604-6620 | Bolt, hex head, long thread, alloy steel |
| NAS6704-6720 | Bolt, hex. head, long thread, cres. |

Table 7-13. Table of nuts.

| Nut Part Number | Description |
| --- | --- |
| AN256 | Nut, self-lock right angle plate |
| AN310 | Nuts, castellated |
| AN315 | Nut, plain |
| AN316 | Nut, check |
| AN320 | Nut, castle shear |
| AN335 | Nut, plain, hex, nonstructural |
| AN340 | Nut, plain, hex., n-s, course thread |
| AN341 | Nut, plain, hex. |

Table 7-13. (continued)

| Nut Part Number | Description |
| --- | --- |
| AN345 | Nut, plain, hex., n-s, fine thread |
| AN350 | Nut, plain, wing |
| AN355 | Nut, engine, slotted |
| AN356 | Nut, stamped |
| AN360 | Nut, plain, engine |
| AN361 | Self-locking nut plate, countersunk 100°, 550 °F. |
| AN362 | Nut, plate, self-locking, noncounters., 550°F. |
| AN363 | Nut, self-locking, 550 °F. |
| AN364 | Nut, self-locking, thin, 250 °F. |
| AN365 | Nut, self-locking 250°F. |
| AN366 | Nut, plate, noncounters., 250°F. |

Table 7-13. (continued)

| Nut Part Number | Description |
| --- | --- |
| AN373 | Countersunk nut, plate 100°, 250°F. |
| AN805 | Nut, union |
| AN817 | Nut, coupling |
| AN818 | Nut, coupling |
| AN924 | Nut, flared tube |
| AN3054 | Nut, coupling, elec. conduit |
| AN3066 | Nut, plain, hex. conduit coupling |
| AN6289 | Nut, flared tube universal fitting |
| AN121501-121550 | Nut, plain or cres. steel |
| AN121551-121600 | Nut, castel., hex. |
| MS9099-9100 | Nut, hex., boss connection, aluminum & cres. |

Table 7-13. (continued)

| Nut Part Number | Description |
| --- | --- |
| MS9197-9199 | Nut, tube coupling |
| MS9200-9201 | Nut, plain, hex., boss connection |
| MS9356-9357 | Nut, plain hex., A-286 |
| MS9358-9359 | Nut, castellated hex., A-286 |
| MS9360 | Nut, plain hex. Drilled, A-286 |
| MS9361-9362 | Nut, plain hex. Check, A-286 |
| MS9363-9364 | Nut, slotted hex. Shear hd., A-286 |
| MS9553 | Nut, hex. Boss connection, cres. |
| MS9766-9767 | Nut, dbl. Hex. Cup washer locked, AMS 5737 cres. And AMS 6322 cad. |
| MS9881 | Nut, plain, hex. AMS 6322, cad. Plate |
| MS9882 | Nut, plain, hex., drilled, AMS 6322, cad. Plate |

Table 7-13. (continued)

| Nut Part Number | Description |
| --- | --- |
| MS9951 | Nut, spanner, end slots, cup washer locked, AMS 6322 |
| MS16203 | Nut, plain, hex. Nonmagnetic |
| MS17825-17826 | Nut, self-locking, castle, hex. Regular and thin |
| MS17828 | Nut, self-locking, nylon insert, 250°, regular ht., monel |
| MS17829-17830 | Nut, self-locking, nylon insert, 250°, regular ht., cres. Steel, steel |
| MS19067-19068 | Nut, plain, round, retaining |
| MS20341 | Nut, electrical, plain, hex. |
| MS20364 | Nut, self-locking, 250 °F, thin |
| MS20365 | Nut, self-locking, 250° F, regular |
| MS20501 | Nut, plate, self-locking, two lug |
| MS21025 | Nut, castellated bearing, retaining |

Table 7-13. (continued)

| Nut Part Number | Description |
| --- | --- |
| MS21047-21048 | Nut, self-locking, plate, two lug, low ht. |
| MS21049-21050 | Nut, self-locking, plate, two lug, 100° csk., low ht. |
| MS21051-21052 | Nut, self-locking, plate, one lug, low ht. |
| MS21053-21054 | Nut, self-locking, plate, one lug, 100° csk. |
| MS21055-21056 | Nut, self-locking, plate, corner, low ht. |
| MS21057-21058 | Nut, self-locking, plate, corner, 100° csk. |
| MS21059-21060 | Nut, self-locking, plate, two lug, floating, low ht. |
| MS21061-21062 | Nut, self-locking, plate, floating low ht., one lug |
| MS21069-21070 | Nut, self-locking, plate, two lug, low ht., reduced rivet spacing |
| MS21071-21072 | Nut, self-locking, plate, one lug, low ht., reduced rivet spacing |
| MS21073-21074 | Nut, self-locking, plate, corner, reduced rivet spacing |

Table 7-13. (continued)

| Nut Part Number | Description |
| --- | --- |
| MS21078 | Nut, self-locking, plate, two lug, nylon insert |
| MS21080 | Nut, self-locking, plate, one lug, nylon insert |
| MS21081 | Nut, self-locking, plate, corner, nylon insert |
| MS21083 | Nut, self-locking, hex., nylon insert |
| MS21340 | Nut, plain, hex., electrical, thin, wire holes |
| MS21917 | Nut, sleeve coupling, flareless |
| MS21921 | Nut, sleeve coupling, flareless |
| MS24679-24680 | Nut, plain cap, low & high crown |
| MS25082 | Nut, plain, thin, hex., electrical |
| MS27040 | Nut, plain square |
| MS27128 | Nut, plain, welding |

Table 7-13. (continued)

| Nut Part Number | Description |
| --- | --- |
| MS27130-27131 | Nut, blind, rivet, flathead., open and closed end |
| MS27151 | Nut, stamped |
| MS27955 | Nut, spanner, plain, round |
| MS35425-35426 | Nut, wing, plain & drilled |
| MS35649-35650 | Nut, plain hex. |
| MS35690-35691 | Nut, plain hex. |
| MS35692 | Nut, slotted hex. |
| MS51967-51972 | Nut, plain, hex. |
| MS90415 | Nut, self-locking, 12 point captive washer |
| MS172236-172270 | Nut, spanner, bearing, retaining |
| MS172321-172370 | Nut, spanner |

Table 7-13. (continued)

| Nut Part Number | Description |
| --- | --- |
| NAS395-396 | Nut, U type |
| NAS443 | Nut, self-locking, int. wrenching |
| NAS444-445 | Nut, double lug, anchor type, offset |
| NAS446 | Nut, flat type |
| NAS447-448 | Nut, plate, self-locking |
| NAS449 | Nut, anchor type |
| NAS450 | Nut, plate, self-locking |
| NAS463 | Shim, plain anchor nut |
| NAS487 | Nut, instrument mount |
| NAS500 | Shim, anchor nut, csk. |
| NAS509 | Nut, drilled |

Table 7-13. (continued)

| Nut Part Number | Description |
| --- | --- |
| NAS577-578 | Nut, self-locking floating barrel retainer |
| NAS671 | Nut, plain hex., small pattern |
| NAS680-681 | Nut, plate, self-locking, two lug |
| NAS682-683 | Nut, plate, self-locking, one lug |
| NAS684-685 | Nut, plate, corner, self-locking |
| NAS686 | Nut, plate, self-locking, two lug, floating |
| NAS687 | Nut, plate, self-locking, one lug |
| NAS688-695 | Nut Assembly, self-locking, gang channel |
| NAS696 | Nut, plate self-locking, one lug, miniature |
| NAS697 | Nut, plate, self-locking, two lug, miniature |
| NAS698 | Nut, plate, corner, self-locking, miniature |

Table 7-13. (continued)

| Nut Part Number | Description |
| --- | --- |
| NAS1021-1022 | Self-locking Nut, hex., regular and low ht. |
| NAS1023-1024 | Nut, plate, self-locking, two lug |
| NAS1025-1026 | Nut, plate, self-locking, one lug |
| NAS1027-1028 | Nut, plate, corner, self-locking |
| NAS1029-1030 | Nut, plate, self-locking, one lug, two lug |
| NAS1031 | Nut, plate, self-locking, two lug, floating |
| NAS1032 | Nut, plate, self-locking, one lug, floating |
| NAS1033 | Nut, plate, right angle, floating, self-locking |
| NAS1034-1041 | Nut Assembly, self-locking, gang channel |
| NAS1067 | Nut, plate, self-locking, one lug, miniature |
| NAS1068 | Nut, plate, floating, self-locking, two lug, miniature |

Table 7-13. (continued)

| Nut Part Number | Description |
| --- | --- |
| NAS1098 | Nut, tube fitting |
| NAS1287-1288 | Nut, hexagonal, self-locking, nut and washer shear pin |
| NAS1291 | Nut, hexagonal, self-locking, low height |
| NAS1329 | Nut, blind rivet, flathead, internal thread |
| NAS1330 | Nut, blind rivet, csk. Head, internal thread |
| NAS1408-1409 | Nut, hexagonal, self-locking, regular height, coarse and fine thr. |
| NAS1410 | Nut, tube fitting |
| NAS1423 | Nut, plain, thin hex., drilled jamnut |
| NAS1473 | Nut, plate, self-locking, two lug, cap floating |
| NAS1474 | Nut, plate, self-locking, two lug, cap floating, reduced rivet spacing |
| NAS1512-1513 | Nut, plate, self-locking gang channel |

Table 7-13. (continued)

| Nut Part Number | Description |
| --- | --- |
| NAS8679 | Nut, self-locking, low height 550 °F, 800 °F |

Table 7-14. Table of washers.

| Washer Number | Description |
| --- | --- |
| AN935 | Washer, lock, spring |
| AN936 | Washer, tooth lock |
| AN950 and 955 | Washer, ball socket, ball seat |
| AN960 | Washer, flat |
| AN961 | Washer, flat electrical (Brass, silver or tin-plated) |
| AN970 | Washer, flat, wood |
| AN975 | Washer, flat |
| AN8013 | Washer, insulator |
| AN122576-122600 | Washer, plain steel |

Table 7-14. (continued)

| Washer Number | Description |
| --- | --- |
| MS9081 | Washer, key dbl. Bearing retaining |
| MS9274 | Washer, key, dbl. Bearing ret. Cres. |
| MS9276 | Washer, key, cres. AMS 5510 180° locking |
| MS9320-9321 | Washer, flat AMS 6350 and 5510 |
| MS9482 | Washer, flat, steel AMS 6437 or AMS 6485 diffused nickel cad. Csk. |
| MS9549 | Washer, flat, AMS 5510 |
| MS9581 | Washer, key, cres. AMS 5510 90° locking |
| MS9582 | Washer, key, cres. AMS 5510 270° locking |
| MS9684 | Washer, cup, lock cres. AMS 5510 |
| MS9768 | Washer, flat, cres. AMS 5525 and AMS 5737 csk. |
| MS9880 | Washer, cup, lock cres. AMS 5510 |

Table 7-14. (continued)

| Washer Number | Description |
| --- | --- |
| MS9952 | Washer, cup, lock, spanner nut, cres. AMS 5646 |
| MS15795 | Washer, flat, metal |
| MS17811 | Washer, thrust, steel |
| MS19069-19070 | Washer, key, retaining |
| MS20002 | Washer, plain, csk., hi-strength |
| MS21258 | Washer, key, retaining, steel |
| MS25081 | Washer, key |
| MS27051 | Washer, slotted |
| MS27111 | Washer, finishing, countersunk |
| MS27129 | Washer, finishing, csk., 80°, 82° |
| MS27183 | Washer, flat, round |

Table 7-14. (continued)

| Washer Number | Description |
| --- | --- |
| MS28777 | Washer, hydraulic, packing backup |
| MS35333-35334 | Washer, lock, flat internal tooth, light and heavy |
| MS35335-35336 | Washer, lock, flat and csk., external tooth |
| MS35337-35340 | Washer, lock, med., light, heavy |
| MS35790 | Washer, lock, 100° csk., ext. tooth |
| MS122026-122075 | Washer, lock spring |
| MS172201-172235 | Washer, key, single bearing, retaining |
| MS172271-172320 | Washer, key, single |
| NAS70 | Washer, plain |
| NAS143 | Washer, csk. and plain |
| NAS390-391 | Washer, finishing |

Table 7-14. (continued)

| Washer Number | Description |
| --- | --- |
| NAS460 | Washer, tab type |
| NAS513 | Washer, rod end locking, steel |
| NAS549 | Washer, flat, resin fiber |
| NAS620 | Washer, flat, reduced O.D. |
| NAS1061 | Washer, hi-temp, lock, spring |
| NAS1099 | Washer, bevel 9 1/2° |
| NAS1169 | Washer, dimpled 100° |
| NAS1197 | Washer, flat, 5052 aluminum |
| NAS1252 | Washer, flat |
| NAS1401 | Washer, radius, al. Alloy, steel, cres. Steel |
| NAS1515 | Washer, plastic and rubber |

Table 7-14. (continued)

| Washer Number | Description |
| --- | --- |
| NAS1587 | Washer, 1200 °F, plain and csk. |
| NAS1598 | Washer, sealing |
| NAS1636 | Washer, key dual tab |
| NAS1640 | Washer, lock, spring, nonmagnetic |

Table 7-15. Table of pins.

| Pin Number | Description |
| --- | --- |
| AN253 | Pin, hinge |
| AN380 & 381 | Pin, cotter |
| AN385 | Pin, plain taper |
| AN386 | Threaded taper pin |
| AN392-406 | Pin, flathead |
| AN415 | Pin, lock |

Table 7-15. (continued)

| Pin Number | Description |
| --- | --- |
| AN416 | Pin, retaining safety |
| AN122676-122775 | Pin, dowel steel |
| AN121601-121925 | Pin, flathead clevis .125 -.500 drilled shank |
| AN150201-150400 | Pin, lock, steel, brass |
| MS9047 | Pin, spring, steel, phoshated finish |
| MS9048 | Pin, spring, steel, cadmium plate |
| MS9105 | Pin, lock |
| MS9164 | Pin, straight, steel, headless, oversize |
| MS9245 | Pin, cotter, cres. AMS 7211 |
| MS9389 | Pin, straight, headless, lock, AMS 5735 |
| MS9390 | Pin, straight, headless, cres. AMS 5735, dowel std. & O.S. |

Table 7-15. (continued)

| Pin Number | Description |
| --- | --- |
| MS9462-9468 | Pin, straight hd. AMS 5735 |
| MS9486 | Pin, straight, headless, lock, AMS 5132 |
| MS9842-9848 | Pin, straight, headed, AMS 5616 |
| MS16555 | Pin, straight, headless .0002 over nominal size |
| MS16556 | Pin, straight, headless .001 over nominal size |
| MS16562 | Pin, spring, tubular, slotted |
| MS17984-17990 | Pin, quick release, positive lock |
| MS20253 | Pin, hinge |
| MS20392 | Pin, straight, headed, drilled |
| MS24665 | Pin, cotter |
| MS35671-35679 | Pin, grooved, headless, tapered groove |

Table 7-15. (continued)

| Pin Number | Description |
| --- | --- |
| MS35810 | Pin, clevis, headed |
| MS39086 | Pin, spring, tubular coiled |
| MS51923 | Pin, spring, tubular coiled |
| MS51987 | Pin, tubular coiled |
| MS171401-171900 | Pin, spring, SS, steel |
| NAS427 | Pin, pulley guard, steel, al. Alloy |
| NAS561 | Pin, spring slotted and coiled, heavy duty |
| NAS574 | Pin, rear mounting |
| NAS607 | Pin, headless, dowel, steel |
| NAS1292-1296 | Pin, shear thread 100° flush head |
| NAS1322 | Pin, shear thread, protruding head |

Table 7-15. (continued)

| Pin Number | Description |
| --- | --- |
| NAS1332-1346 | Pin, quick release |
| NAS1353-1366 | Pin, quick release, positive locking, double acting |
| NAS1407 | Pin, spring, coiled |
| NAS1436-1442 | Pin, swage lock, 100° shear head, pull type, steel |
| NAS1446-1452 | Pin, swage lock, protruding head, pull type, steel |
| NAS1456-1462 | Pin, swage lock, 100° head, pull type, steel |
| NAS1465-1472 | Pin, swage lock, protruding head, pull type, steel |
| NAS1475-1482 | Pin, swage lock, 100° head, pull type, steel |
| NAS1486-1492 | Pin, swage lock, 100° head, stump type, steel |
| NAS1496-1502 | Pin, swage lock, protruding head, stump type, steel |
| NAS1525-1532 | Pin, swage lock, protruding head, al. alloy |

Table 7-15. (continued)

| Pin Number | Description |
| --- | --- |
| NAS1535-1542 | Pin, swage lock, 100° hd., tension, pull type, alum. |
| NAS1546-1552 | Pin, swage lock, 100° hd., tension, stump type, alum. |
| NAS1583 | Pin, 100° csk. hd., hi-shear rivet, 1200 °F |
| NAS1584 | Pin, flathead, hi-shear rivet, 1200 °F |

7-196.¾7-206. [RESERVED.]